C182Q N7569S (6/24)

Emergency checklist

Procedures shown in bold faced type are immediate action items which should be committed to memory.

For amplified emergency procedures refer to POH Sec 3.

Transponder Codes:

7700 – Emergency

7600 – Lost Communications

7500 – Hijack

Emergency Frequency – 121.5

ENGINE FAILURES

DURING FLIGHT (RESTART)

IF TIME PERMITS

- Airspeed – 70 KIAS Flaps Up

- Landing Site – SELECT

- Carb Heat – ON

- Fuel Selector Valve – BOTH

- Mixture – RICH

- Throttle – SET

- Primer – LOCKED

- MAGS – L&R, Both, Start

FORCED LANDING

IF ENGINE FAILS TO RESTART

- Airspeed – 70 KIAS (Flaps Up)

- Fuel Selector Valve – OFF

- Mixture – IDLE CUT OFF

- MAGS – OFF

- Cabin – SECURE

- Declare emergency

- Flaps – AS REQUIRED

- Cabin Doors – UNLATCH

- Master Switch – OFF Before Touchdown

- Touchdown – TAIL LOW

- Brakes – APPLY HEAVILy

FIRES

DURING START ON GROUND

- MAGS – START

Continue cranking

IF ENGINE STARTS

- Power – 1700 RPM 2-3 MIN

- Engine – SHUTDOWN /

INSPECT FOR DAMAGE

IF ENGINE FAILS TO START

- Throttle – FULL OPEN

- Mixture – Idle CutOff

- MAGS – START

Continue cranking

- Fuel Selector Valve – OFF

- Engine – SECURE

A. Master Switch – Off

B.  MAGS – OFF

- Parking Brake – RELEASE

- Fire Extinguisher – OBTAIN

- Airplane – EVACUATE

- Fire – EXTINGUISH

- Fire Damage – INSPECT

ELECTRICAL FIRE IN FLIGHT

- Master Switch – Off

- Vents/Cabin Air/Heat – Closed

- Fire Extinguisher – ACTIVATE

- Radio Master – OFF

- Electrical Switches – OFF

- VERIFY FIRE IS EXTINGUISHED

- Vents/Cabin Air/Heat – OPEN

- Master Switch – ON (As Req.)

- Circuit Breakers – CHECK

DO NOT RESET

- Radio Master – ON (As Req.)

- Windows – OPEN (As Req.)

- Land – At Nearest Suitable Airport

CABIN FIRE

- Master Switch – Off

- Vents/Cabin Air/Heat – Closed

- Fire Extinguisher – ACTIVATE

- VERIFY FIRE IS EXTINGUISHED

- Vents/Cabin Air/Heat – OPEN

- Windows – OPEN (As Req.)

- Land – At Nearest Suitable Airport

WING FIRE

- NAV and Strobe Light Switch – OFF

- PITOT HEAT Switch – OFF

- Sideslip flames away from fuel tank and cabin.

- VERIFY FIRE IS EXTINGUISHED

- lAND – ASAP

- Flaps – ONLY AS REQUIRED ON FINAL APPROACH (AVOID IF POSSIBLE)

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUTOFF

- Fuel Selector Valve – OFF

- Master Switch – OFF

- Cabin Heat & Air – OFF

- Overhead Vents – OPEN

- Windows – OPEN (As Req.)

- Airspeed – AT LEAST 100 KIAS

- Forced Landing – EXECUTE

ROUGH RUNNING ENGINE OR PARTIAL LOSS OF POWER

SPARK PLUG FOULING

SLIGHT ENGINE ROUGHNESS

- Carb Heat – ON

- MAGS – CHECK (R&L)

- Mixture – LEAN FOR CRUISE        (SEE POH)

- Mixture – Enrich if Problem Persists

- Land – At Nearest Suitable Airport

MAGNETO MALFUNCTION

SUDDEN ENGINE ROUGHNESS

- MAGS – CHECK (R&L)

- Mixture – FULL RICH

- Throttle – SET (As Req.)

- If Persists – Switch to Good MAG

- Land – At Nearest Suitable Airport

LOW OIL PRESSURE

OIL PRESS ANNUNCIATOR

- OIL PRES Indicator – CHECK

- OIL TEMP Indicator – CHECK

IF BOTH INDICATING NORMAL

- Land – At Nearest Suitable Airport

IF INDICATING LOSS OF OIL PRESSURE AND RISE IN OIL TEMP

- Power – REDUCE IMMEDIATELY

- Forced Landing – EXECUTE