C182Q N7569S

checklist (1/26)

PRE-FLIGHT CABIN

- Tach - Record

- Control Lock & Pitot Cover – Remove

- AROW Documents – Check

- Fire Extinguisher – Green

- Fuel Selector – Both (L/R During Fueling)

- MAGS – Off / Key Out

- Radio Master – Off

- Master Switch – On

- Flaps – Down

- Fuel Quantity – Check

- Radio Master – ON/Check Fan/OFF

  - Ext. Lights/Stall Horn/Pitot Heat – Check

  - Ext Lights/Pitot Heat – Off

  - Master Switch – Off

PRE-FLIGHT EMPENNAGE

- Baggage Door – Secured and Locked

- Autopilot Static Source (2) – Clear

- Tie Down – Remove

- Surfaces & Controls – Check

- Antennas – Check

PRE-FLIGHT RIGHT WING

- Surfaces & Controls – Check

- Tie Down / Chocks – Remove

- Strut / Tire / Brake – Check

- Fuel Sump – Drain

- Fuel Quantity – Check Visually

- Fuel Cap – Secure

PRE-FLIGHT NOSE

- Static Source – Check

- Strut and Tire – Check

- Propeller / Spinner – Check

- Cowling/Intakes – Check

- Air Filter – Check

- Oil Filler Cap – Secure

- Engine Oil – Check (Min 9 quarts)

- Fuel Strainer – Drain

- Static Source – Check

PRE-FLIGHT LEFT WING

- Pitot Tube – Check

- Fuel Vent – Check

- Stall Horn – Check

- Surfaces & Controls – Check

- Tie Down / Chocks – Remove

- Strut / Tire / Brake – Check

- Fuel Sump – Drain

- Fuel Quantity – Check Visually

- Fuel Cap – Secure

BEFORE STARTING ENGINE

- Preflight Inspection – Complete

- Passenger/Pilot Briefing – Complete

- Seats & Seatbelts – Adjust & Lock

- Cowl Flaps – Open

- Fuel Selector Valve – Both

- Alternate Static Source – Check then Off

- Circuit Breakers – In (3 locations)

- Electrical Equipment – Off

- Radio Master – Off

- Autopilot/Trim - Off

STARTING ENGINE

- Brakes – Set

- Rotating Beacon – On

- Propeller – Visually Clear

- Master Switch – On

- Mixture – Full Rich

- Propeller – High RPM

- Throttle – Open ½ inch

- Oil Temp – Check Cold or Hot

- Prime – Strokes (Cold 4-6 / HOT 1-2)

- Primer – Locked

- Propeller – Verbally “CLEAR!”

- Start Time – Recorded

  - Magnetos – Start then Both

  - Throttle – 1000 RPM

  - Oil Pressure – Check

  - Ammeter – POS. Charge (+)

  - Mixture – Lean 1 inch

PRE-TAXI

- Fuel Totalizer – Set (As Req.)

- Radio Master– On

- NAV Lights – On (As Req.)

- Flaps – Retract

- ATIS / AWOS – Obtained

- Altimeter – Set G5 and Analog

- Flight Instruments – Set and Check

- Garmin GTN650

  - Database – Check

  - G5 HSI – Check

- Garmin 696

  - Database – Check

  - Traffic – Enabled

- VOR Test – Within 30 Days (IFR)

- Taxi Light – As Required

- Clearance – Obtained

- Transponder – Code Set, Alt Mode

- Autopilot & Trim – On

TAXI

- Brakes – Test (L/R/BOTH)

- Flight Controls – Wind Correction Set

- Instrument Check – IFR / VFR Check

- Mixture – Rich (As Required)

ENGINE RUNUP

- Brakes – Set

- Mixture – Rich

- Flight Controls – Free & Correct

- Elevator Electric Trim – Test

- Elevator/ Rudder Trim – Set for Takeoff

- Throttle – 1700 RPM

- Magnetos – (R & L – 150/50)

- Propeller – Cycle RPM (3-5 Times)

- Engine Ind. – Check

- Ammeter – Check POS Charge

- Carb Heat – On and Check

- Idle RPM – Check (575-675 RPM)

- Carb Heat – Off

- Throttle – 1000 RPM

- Throttle Friction – Set

- COM / NAV – Set

- Flaps – Set (0°-20°)

- Parking Brake – Check Off

BEFORE TAKEOFF

- Windows and Doors – Closed & Latched

- Fuel Selector Valve – Verify Both

- Engine Instruments – Check

- Carb Heat – Verify Off

- Transponder – Verify Code Set, Alt Mode

- Strobes / Landing Light – On

- Cowl Flaps – Verify Open

- Departure Briefing – Complete

  - Runway – Confirm Direction

  - Crosswinds – Confirm Direction

  - Emergency Plan – Verbal Review

- Takeoff Time – Recorded

CRUISE CLIMB

- Airspeed – 85-95 KIAS

- Power – 23” & 2400 RPM

- Mixture – Rich (Lean above 5,000 ft)

- Flaps – Retract

- Engine Instruments – Check

- Landing Light – Off

CRUISE

  - Power – 15”-23” 2000-2400 RPM

Crosswind Max Demonstrated – 15

Vr – 50-60

Vx – 54

Vy – 78

Vso – 45

Vs – 48

Best Glide – 70

Va – 1950 / 2450 / 2950 lbs – 89 / 100 / 111

Vno – 143

Vne – 179

Vfe (10°) -140 / (20° - Full) 95

  - Mixture – Lean (100° Rich of EGT Peak)

  - Engine Instruments – Check

  - Cowl Flaps – Closed (As Req.)

DESCENT / APPROACH

- ATIS / AWOS – Obtained

- Altimeter – Set G5 and Analog

- Pre-Landing Brief – Complete

- Com / Nav – Set

- Carb Heat – As Required

- Mixture – Full Rich

- Landing Light – On

BEFORE LANDING

- Cowl Flaps – Open

- Flaps – As Required

- Propeller – High RPM

- Mixture – Rich

- Carb Heat – On (As Req.)

- Autopilot – Disengage

- Trim – As Required

AFTER LANDING - (Clear of Runway)

- Flaps – Retract

- Carb Heat – Off

- Pitot Heat – Off

- Landing/Taxi Light – As Req.

- Strobe Lights – Off

- Mixture – Lean 1”

SHUTDOWN

- Transponder – VFR (1200)

- Throttle – 1000 RPM

- Elevator Trip -- Neutral

- Autopilot/Trim – Off

- Radio Master – Off

- Electrical Switches – Off (Beacon ON)

- Mixture – Idle Cutoff

- MAGS – Off and Key Removed

- Alternator & Master Switch – Off

- Fuel Selector – Left, Right, or Off

- Tach / MX Squawks – Record

- Oil Change – Notify if Due Soon

- Control Lock / Pitot Cover – Install

- Chocks / Tie Downs – As Req.

- Flight Plan – CLOSED!

KCMI Frequencies

ATIS – 124.85	Clearance – 128.75

Ground – 121.8	Tower – 120.4

Approach/Departure – 121.35 (North/East)

Approach/Departure – 132.85 (South/West)

CMI VOR – 115.75